Purpose

- Make exposure visible, for one product family or one lane.

- Use it to feed the risk register and the monthly risk review.

- Keep it to one A3 page and readable.

When to use

- At the start of risk work for a site or family.

- Before a major change (new supplier, new route, new line).

- After a disruption to learn and adjust.

- When service is unstable and you need a clear picture.

What you need (inputs)

- product family/flow

- SOPs/line layout

- key assets

- staffing coverage by shift

- suppliers & alternates (T1/T2 that truly matter)

- IT/OT systems (MES/PLC/WMS/ERP)

- utilities (power, compressed air, network)

- recent downtime/quality issues.

Prep

- Pick the scope: one product family or one transport lane.

- Gather facts: current lead times, carrier mix, recent shipment list, Tier-1 suppliers, the few Tier-2 that matter most, known single points of failure, your constraint line.

- Agree who will own updates.

Header

| Product/Flow | Scope (from → to) | Date / Version |
| --- | --- | --- |
| Site/Area | Owner | Team |

Build the map

- Left to right chain (column headers): Tier-2 (critical) → Tier-1 → Inbound legs (carrier, mode, gateway) → Site/lines (constraint marked) → Outbound legs/gateways → Key customers.

- Lay out the horizontal lanes: Material flow | Core processes/lines/cells | People/skills | IT/OT systems | Utilities | External partners.

- Plot the main flow: In Core processes, show steps (receive → store → prep → make → pack → ship). Connect left→right. Mark the constraint clearly.

- Add buffers & decouplers: In Material: inventory triangles/boxes; annotate hours of cover at average rate.

- Add alternates & bypasses: Parallel line, alternate supplier, manual fallback. If no alternate, tag the upstream step as single point of failure. Mark SPOFs with a simple symbol.

- Overlay enablers: People (unique/single-person coverage), IT/OT (MES/PLC/WMS/ERP + interfaces), Utilities (power/air/steam/network), External partners.

- Controls & detection: Interlocks, QA holds, alarms/reconciliations; note how failure is detected (alarm/SPC/mismatch).

- Early signals: Next to each potential SPOF, list practical, observable signals (no new tools required).

- Mini recovery cards: For major nodes, capture: Owner • Manual fallback? • Spare on-hand? • MTTR • Vendor SLA • Alternate path? (None → likely SPOF.)

- Tag risks: Add the Risk ID from the Register; color border by Priority (High=Red / Medium=Amber / Low=Green).

- Validate fast: 45-min huddle (ops, maintenance, QA, IT, logistics). Confirm buffers, alternatives, recovery cards. Capture top 5 SPOFs and owners.

What to put in each area

- Tier-2 (that truly matter): unique processes, sole tooling, special materials, utilities-as-a-service. Add city or region and any long qualification lead times.

- Tier-1 suppliers: name the site, not only the company. Note typical lead time if it drives risk.

- Inbound legs: carrier, mode, transit time, main gateway (port, airport, border). Add the tested alternative if it exists.

- Site or lines: the lines that carry the volume. Mark the constraint. Note critical spares and any unique tooling.

- Buffers/decouplers: Where inventory sits and hours of cover (tie to the Material lane annotation).

- Utilities: power, compressed air, process water, steam, backup generation (and any single-feed exposure).

- IT systems: the few you cannot run or ship without (MES, WMS, label printing, SCADA, authentication, database). Note simple dependencies in plain words.

- Unique skills or roles: any single-person risk. Add named deputies if you have them.

- Outbound legs or gateways: carrier, mode, main gateway, tested alternative.

- Key customers: only those that drive priority. Add a short tag such as “OTIF ≥ 98%”.

Early signals to consider

Pick what you can see early and consistently.

- Suppliers: QC pass trend, late or missing ASNs, sudden lead-time changes, local labor or safety news.

- Inbound: roll-overs, dwell time, hub congestion notices, border delays.

- Site: vibration or temperature on a critical asset, scrap spikes, maintenance backlog on the constraint.

- Utilities: pressure or voltage alarms, failed generator test, water quality alerts.

- IT: failed backup, unusual login failures, antivirus alerts, repeated label print errors.

- People: overtime spikes, absenteeism on a key shift, holiday overlap in a unique role.

Symbol conventions / color legend (keep minimal)

- Process step = rounded rectangle

- Buffer/stock = triangle/box (“hrs cover”)

- SPOF tag = red triangle “SPOF”

- Risk ID = small “R-####” (border colored by priority)

- Alternate path = dashed line

- Control/detection = shield or “CTRL”

- People/skill = person icon + skill (e.g., “only CNC L3”)

- IT/OT = rectangle with system (MES/WMS/PLC)

- Utilities = line with icons; mark single feed

- External = hexagon with vendor/carrier.

What to look for (SPOF checklist)

- Material/partners:

  - single-source item

  - MOQ/lead time > buffer

  - sole customs broker/carrier

- Equipment:

  - one machine with no bypass; long changeover

  - unique tooling

  - no spare critical part.

- People:

  - single skill holder per shift

  - unplanned leave = stop

- IT/OT:

  - one database/server

  - fragile interface (manual rekey)

  - Wi-Fi dead zone.

- Utilities:

  - single power feed

  - compressor at capacity; steam/air header without loop

  - no UPS.

- Controls:

  - detection late/manual

  - alarms without owner

  - bypass culture.

- Logistics:

  - one dock

  - one lane

  - one slotting strategy that breaks on peak.

Build the priority risk list panel (5–10 minutes)

- Choose 3 to 6 items that sit above your agreed risk threshold.

- For each: risk, owner, next action with a date.

- Everything else goes into the risk register.

Findings summary (paste under the map)

| # | Node / Dependency | Why SPOF | Current cover | Quick mitigate (2–4 wks) | Structural fix (1–3 qtrs) | Owner | Target |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Infeed palletizer L2 | No bypass and no buffer | 0.5 h | Add 2h WIP after mixer | Install diverter to Line 1 | Ops Lead | 30 Nov |
| 2 | Vision station | Only one trained maint. | n/a | Train backup; document reset | Cross-train 2 techs | Maint | 15 Dec |

Validate the map (5 minutes)

- One page only, readable when printed.

- Single points of failure are marked.

- Alternatives are named where they exist (second source, alternative gateway).

- Early signals are practical and observable.

- Priority list has owners and dates.

Keep it current

- Place the map near the team board. Date it.

- Update monthly, or when anything important changes.

- Use it in the monthly risk review to confirm status, add new exposures, and move items between the watch list and the priority list.

Variants by operation

- Manufacturing: emphasise tooling ownership and transfer rights, spares and PM on the constraint, QA release flow and label printing.

- Logistics or DC: emphasise carrier mix, alternative gateway and backup carrier, parcel versus pallet flow, WMS, sortation, dock capacity.

- Multi-site network: one map per major site. If needed, keep a simple network overlay showing the few cross-site corridors and shared single points of failure.

Watchouts

- Trying to draw the whole company. Keep the scope narrow.

- Listing every supplier and every system. Show only what can stop flow.

- Missing the early signals. Then you may only learn when the line stops.

- Old prints on the wall. Date it and update it.

- No owners or dates on the priority list.