- Design for Assembly (DFA)

- Design for Manufacturing (DFM proper)

- Design for Testing (DFT)

- Design for Logistics (DFL)

- Design for Service / Maintenance (DFS)

Design for Assembly (DFA)

☐  Can parts be assembled with standard tools (no special fixtures unless justified)?

☐  Are fasteners minimized and standardized (e.g., same screw type/size)?

☐  Are parts oriented for easy insertion, without flipping or awkward motions?

☐  Can sub-assemblies be built independently to shorten main line time?

☐  Are part geometries simple enough to reduce assembly complexity?

☐  Is the product design tolerant to minor assembly variation?

☐  Are connectors, cables, or hoses easy to route and secure?

Design for Manufacturing (DFM proper)

☐  Are tolerances realistic for chosen processes (not tighter than needed)?

☐  Are materials compatible with standard equipment and suppliers?

☐  Are critical dimensions accessible for machining, molding, or cutting?

☐  Are surface finishes and coatings specified only where they add value?

☐  Is wall thickness uniform to avoid warping (in molding/casting)?

☐  Are undercuts, sharp corners, or thin ribs avoided unless essential?

☐  Are parts designed for standard batch sizes and machine capacities?

☐  Are parts designed using standard, off-the-shelf components where possible ?

☐  Are materials and parts readily available from multiple suppliers?

☐  Are features designed to self-locate to reduce jig/fixture reliance?

Design for Testing (DFT)

☐  Are test points included and accessible without disassembly?

☐  Can key functions be tested with simple tools or gauges?

☐  Are electrical test pads exposed and well-placed (for PCBs)?

☐  Are potential failure modes identified, and test coverage ensured?

☐  Is software/firmware testable separately from hardware?

☐  Are inspection methods defined for critical-to-quality features?

Design for Logistics (DFL)

☐  Is the product robust enough for packing, shipping, and handling?

☐  Are pack sizes, pallet height, and stacking strength practical?

☐  Does packaging minimize wasted volume and unnecessary materials?

☐  Can barcodes or labels be placed where they scan cleanly?

☐  Is packaging compliant with customer/regional requirements?

☐  Can mixed-SKU pallets be built efficiently if required?

☐  Is packaging recyclable or reusable where possible?

Design for Service / Maintenance (DFS)

☐  Can wear parts be replaced without dismantling the whole unit?

☐  Are hazardous materials easy to identify and handle?

☐  Are manuals or one-point lessons (OPLs) defined for key service tasks?

☐  Are fasteners and connectors designed for easy access in service?

☐  Is diagnostic information available to speed up troubleshooting?

☐  Are critical spare parts standardized across models where possible?